Lab01_Journal_Basic-Switching-VLANs.md

Lab Journal — Lab 1

Title: Basic Switch Setup, VLANs, and Access Ports
Environment: Cisco Packet Tracer
Device(s): Cisco 2960 Switch (S1), 4 PCs
Author: Marc-Anthony Jones
Date: December 28, 2025

1. Lab Objective

The objective of this lab was to configure a Layer 2 switch with two VLANs (VLAN10 and VLAN20), assign access ports to each VLAN, and verify that hosts in different VLANs are isolated in the absence of Layer 3 routing.

2. Initial Issues Encountered

Several issues were encountered during validation rather than configuration:

- PCs initially did not have static IP addresses configured. Because no DHCP server or router existed in the topology, hosts were unable to communicate until IPs were manually assigned.

- During testing, IP addresses were mistakenly assigned to the wrong PCs, which caused same-VLAN ping failures even though the switch configuration was correct.

- Early in the lab, interface verification was confusing because some PCs were not yet physically connected to the switch ports.

- Minor CLI errors occurred due to incorrect command spelling (e.g., how instead of show), which reinforced the importance of command accuracy.

All issues were resolved by correcting cabling, re-entering static IP addresses on each PC, and re-running verification commands.

3. Research & Concept Review

To strengthen understanding and avoid configuration errors, the following key terms and commands were reviewed:

- no ip domain-lookup — Prevents the switch from attempting DNS resolution when invalid commands are entered, improving CLI usability.

- switchport mode access — Forces switch ports into access mode to carry traffic for a single VLAN and prevents unintended trunk negotiation.

- spanning-tree portfast — Allows access ports to immediately transition to the forwarding state, reducing connection delays for end devices.

- Spanning Tree Protocol (STP) — Prevents Layer 2 loops by blocking redundant paths while preserving them for failover.

- Static IP addressing — Required for host communication in the absence of DHCP or Layer 3 devices.

This review ensured commands were applied intentionally and aligned with best practices for access-layer switching.

4. Configuration Summary

The switch was configured with a hostname and basic management settings. Two VLANs were created and named to reflect their purpose. Access ports were assigned to each VLAN and configured with PortFast to optimize end-device connectivity.

VLAN Configuration:

- VLAN10 — USERS (Fa0/1–Fa0/2)

- VLAN20 — ADMIN (Fa0/3–Fa0/4)

Access Port Configuration:

- Ports configured as static access ports

- VLANs assigned appropriately

- PortFast enabled on all end-device ports

5. Verification & Validation

The following commands were used to confirm correct configuration and operational status:

- show vlan brief — Verified VLAN creation and port assignments

- show interfaces status — Confirmed physical connectivity and VLAN association

- show interfaces switchport — Validated access mode operation

- show spanning-tree vlan 10 / 20 — Confirmed ports were forwarding and no Layer 2 loops existed

- show mac address-table dynamic — Verified MAC learning per VLAN

After assigning correct static IP addresses to the PCs, ICMP testing was performed:

- Same-VLAN pings (VLAN10 to VLAN10) succeeded

- Cross-VLAN pings (VLAN10 to VLAN20) failed, as expected due to the absence of Layer 3 routing

This confirmed proper VLAN segmentation and isolation.

6. Outcome

All lab objectives were successfully met. VLANs were correctly configured, access ports were properly assigned, and traffic isolation between VLAN10 and VLAN20 was validated through switch-side verification and host-based testing.

7. Lessons Learned

- Static IP addressing is required for host communication when no DHCP or router is present.

- Incorrect IP assignment can mimic switching or VLAN failures even when the network is correctly configured.

- Physical connectivity must be verified before troubleshooting logical configurations.

- Packet Tracer may retain stale host state; re-entering IP information can resolve unexpected behavior.

- Verification commands are essential for confirming both correctness and operational behavior.

8. Next Steps

Future labs will introduce Layer 3 routing to enable controlled inter-VLAN communication using a router or multilayer switch.